This guide specification is written according to the Construction Specifications Institute (CSI) formats, including MasterFormat, SectionFormat, and PageFormat.

Carefully review and edit this section to meet the requirements of the project and local building code. Coordinate this section with other specification sections and drawings.

Delete all "Specifier Notes" when done editing this section.

SECTION 26 27 26.33

FURNITURE POWER DISTRIBUTION UNITS

- GENERAL

      - CONDITIONS AND REQUIREMENTS

        - The General Conditions, Supplementary Conditions, and Division 01 – General Requirements apply.

      - SECTION INCLUDES

        - Furniture power centers for power and charging.

      - RELATED REQUIREMENTS

Specifier Note: In this article, specify work specified in other sections that is related to work of this section.

        - Division 26 - Electrical: Electrical systems and components.

Specifier Note: The following paragraph is a sample that may be used in this article.  Add to or delete from the following as appropriate for the specific project.

        - Section [xx xx xx] – [Section Title]: [Include brief description of work specified in another section that is related to the work of this section.]

      - REFERENCE STANDARDS

        - Federal Communication Commission (FCC):

          - FCC Part 15, Subpart B – Radio Frequency Devices, Unintentional Radiator.

        - Underwriters Laboratories (UL):

          - UL 962A-18 – Furniture Power Distribution Units.

      - SUBMITTALS

Specifier Note: In this article, specify various types of data to be furnished by the contractor before, during, or after construction. Topics included in this article are product data, shop drawings, samples, design data, test reports, certificates, manufacturers’ instructions, manufacturers’ field reports, qualification statements, and closeout submittals.

        - Submit under provisions of Section [01 33 00] [______].

        - Product Data: Submit for furniture power centers.

      - QUALITY ASSURANCE

Specifier Note: In this article, describe qualifications, regulatory requirements, certifications, field samples, mock-ups, and pre-installation meetings.

Specifier Note: In the following paragraph, select one of the options and delete the other. Verify with manufacturer before selecting one option.

        - Manufacturer Qualifications: Firms regularly engaged in manufacture of furniture power centers of the types and sizes required, whose products have been in satisfactory use in similar service for not less than [five] [10] [_____] years. Provide furniture power centers produced by a manufacturer listed in this section.

        - Source Limitations: Obtain each type of furniture power center through one (1) source from a single manufacturer.

        - Comply with requirements of applicable local codes, NEC, UL, ETL, and NEMA Standards pertaining to furniture power centers.

          - Comply with UL 962A for spill water protection.

          - Comply with FCC Part 15, Subpart B.

      - DELIVERY, STORAGE AND HANDLING

        - Deliver furniture power centers in factory labeled packages.

        - Store and handle in strict compliance with manufacturer’s written instructions and recommendations.

        - Protect from damage due to weather, excessive temperature, and construction operations.

- PRODUCTS

      - ACCEPTABLE MANUFACTURERS

        - Basis-of-Design Product: The design for furniture power distribution units is based on ModPower™ Modular Power System Devices manufactured by Legrand/Wiremold, 60 Woodlawn Street, West Hartford, CT 06110; toll-free 877-295-3472, telephone 860-233-6251, fax 860-232-2062; Web Site: www.legrand.us/wiremold.

        - Substitutions will be considered under provisions of Section 01 25 00.

      - FURNITURE POWER CENTERS

        - Classification and Use: Furniture power centers shall have been examined, tested, and listed by UL to US and Canadian Safety Standards.

          - UL Listed to UL 962A Supplement SD – Furniture Power Distribution Units for Portable (Moveable) Work Space Tables.

        - Furniture Power Centers, General: Furniture power centers shall provide convenience outlets for power and USB charging.

        - Furniture Power Centers:

Specifier Note: Retain one or more of the following subparagraphs as required for a project; delete those not required.

Specifier Note: Retain one of the finish colors in the following subparagraphs and delete the other one.

          - In Surface Primary Unit: Catalog No. MDSF10-[WH][MG]; [white] [magnesium] color; provide unit with one (1) 10-foot cord with a NEMA plug and one (1) 3-foot cord with a connector; equip unit with three (3) 15A tamper resistant receptacles, one (1) USB A, one (1) USB C port, and a circuit breaker. Unit comes with a mounting box to sit flush in table and four (4) cord clips to assist with cable management.

          - On Surface Primary Unit: Catalog No. MDSA10[WH][MG]; [white] [magnesium] color; provide unit with one (1) 10-foot cord with a NEMA plug and one (1) 3-foot cord with a connector; equip unit with three (3) 15A tamper resistant receptacles, one (1) USB A, one (1) USB C port, and a circuit breaker. Unit comes with a clamp and three (3) extra brackets to accommodate various mounting methods and four (4) cord clips to assist with cable management.

          - In Surface Middle Unit: Catalog No. MDCF3[WH][MG]; [white] [magnesium] color; provide unit with two (2) 3-foot cords with a connector at their ends; equip unit with three (3) 15A tamper resistant receptacles, one (1) USB A, one (1) USB C port, and a circuit breaker. Unit comes with a mounting box to sit flush in table and four (4) cord clips to assist with cable management.

          - On Surface Middle Unit: Catalog No. MDCA3[WH][MG]; [white] [magnesium] color; provide unit with two (2) 3-foot cords with a connector at their ends; equip unit with three (3) 15A tamper resistant receptacles, one (1) USB A, one (1) USB C port, and a circuit breaker. Unit comes with a clamp and three (3) extra brackets to accommodate various mounting methods and four (4) cord clips to assist with cable management.

          - In Surface End Unit: Catalog No. MDEF3-[WH][MG]; [white] [magnesium] color; provide unit with one (1) 3-foot cord with a connector at its end to finish the system; equip unit with three (3) 15A tamper resistant receptacles, one (1) USB A, one (1) USB C port, and a circuit breaker. Unit comes with a mounting box to sit flush in table and two (2) cord clips to assist with cable management.

          - On Surface End Unit: Catalog No. MDEA3-[WH][MG]; [white] [magnesium] color; provide unit with one (1) 3-foot cord with a connector at its end to finish the system; equip unit with three (3) 15A tamper resistant receptacles, one (1) USB A, one (1) USB C port, and a circuit breaker. Unit comes with a clamp and three (3) extra brackets to accommodate various mounting methods and two (2) cord clips to assist with cable management.

      - ACCESSORIES

Specifier Note: Retain one or more of the following subparagraphs as required for a project; delete those not required.

        - Cord Clips: Catalog No. MDLCIPS; replacement bag of 10 mounting clips for cord management.

Specifier Note: Retain one of the finish colors in the following subparagraph and delete the other one.

        - On Surface Mounting Brackets: Catalog No. MDBKT-[WH][MG]; [white] [magnesium] color; replacement brackets for on surface mounting units.

- EXECUTION

      - EXAMINATION

Specifier Note: Retain one (1) of the options in the first paragraph below and delete the other one.

        - Examine conditions under which furniture power centers are to be installed. Notify the [Architect/Engineer] [Construction Manager] in writing of conditions detrimental to proper completion of the work. Do not proceed with work until unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Strictly comply with manufacturer's installation instructions and recommendations. Coordinate installation with adjacent work to ensure proper clearances and to prevent electrical hazards.

      - CLEANING AND PROTECTION

        - Clean exposed surfaces using non-abrasive materials and methods recommended by manufacturer.

        - Protect furniture power centers until acceptance.

END OF SECTION

©2021 Legrand/Wiremold. All Rights Reserved.

A copyright license to reproduce this specification is hereby granted

to non-manufacturing engineers, architects, and specification writers.

New 06/21