This guide specification is written according to the Construction Specifications Institute (CSI) formats, including MasterFormat, SectionFormat, and PageFormat.

Carefully review and edit this section to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and drawings.

Delete all "Specifier Notes" when done editing this section.

SECTION 26 05 33.16

FLOOR BOXES FOR ELECTRICAL SYSTEMS

- GENERAL

      - CONDITIONS AND REQUIREMENTS

        - The General Conditions, Supplementary Conditions, and Division 01 – General Requirements apply.

      - SECTION INCLUDES

        - Resource RFBA recessed floor boxes with rectangular activations.

      - RELATED SECTIONS

Specifier Note: In this article, specify work specified in other sections that is related to work of this section.

        - Division 26 - Electrical: Electrical systems and components.

        - Division 27 - Communications: Communications systems and components.

        - Division 28 - Electronic Safety and Security: Security systems and components.

Specifier Note: The following paragraph is a sample that may be used in this article.  Add to or delete from the following as appropriate for the specific project.

        - Section [xx xx xx] – [Section Title]: [Include brief description of work specified in another section that is related to the work of this section.]

      - SUBMITTALS

Specifier Note: In this article, specify various types of data to be furnished by the contractor before, during, or after construction. Topics included in this article are:  product data, shop drawings, samples, design data, test reports, certificates, manufacturers’ instructions, manufacturers’ field reports, qualification statements, and closeout submittals.

        - Submit under provisions of Section [01 33 00] [______].

        - Product Data: Submit for floor boxes.

Specifier Note: Delete the following article if not required for a project.

        - Shop Drawings: For the following electrical system components. Include plans, elevations, sections, details, and attachments to other work.

          - Floor boxes.

      - QUALITY ASSURANCE

Specifier Note: In this article, describe qualifications, regulatory requirements, certifications, field samples, mock-ups, and pre-installation meetings.

        - Manufacturer Qualifications: Firms regularly engaged in manufacture of floor boxes of the types and sizes required, whose products have been in satisfactory use in similar service for not less than 10 years. Provide floor boxes produced by a manufacturer listed in this section.

        - Source Limitations: Obtain each type of floor box through one (1) source from a single manufacturer.

        - Electrical Raceways and Components: Comply with requirements of applicable local codes, NEC, UL, and NEMA Standards pertaining to raceways and components. Listed and labeled in accordance with NFPA 70, Article 100.

        - Floor box assembly shall be investigated and approved by a Nationally Recognized Testing Laboratory for suitability in the country of installation. Investigation shall include:

          - Scrub water exclusion test per UL891 and UL514A, Section 32A.2.

          - Compliance with NFPA 70 (NEC).

          - cULus Listed as metallic outlet boxes for tile, terrazzo, carpet, and wood floors; File E2961, Guide QCIT.

      - DELIVERY, STORAGE AND HANDLING

        - Deliver floor boxes and fittings in factory labeled packages.

        - Store and handle in strict compliance with manufacturer’s written instructions and recommendations.

        - Protect from damage due to weather, excessive temperature, and construction operations.

      - WARRANTY

        - Manufacturer's Warranty: Manufacturer's standard form in which manufacturer agrees to replace floor boxes that are found to be defective during the warranty period.

          - Warranty Period: [One (1)] [Insert number] years from date of final acceptance.

- PRODUCTS

      - ACCEPTABLE MANUFACTURER

        - Basis-of-Design Product: The design for recessed floor boxes and fittings is based on the Resource RFBA Floor Box Series manufactured by Legrand/Wiremold, 60 Woodlawn Street, West Hartford, CT 06110; telephone 877-295-3472; Web Site: www.legrand.us/wiremold.

        - Substitutions will be considered under provisions of Section 01 25 00.

Specifier Note: In the following article, retain only those products that are to be incorporated into the project and delete the others.

      - RECESSED FLOOR BOXES

        - RFBA2R25 Series 2-Gang Above Grade Shallow Depth Floor Boxes with Rectangular Activations: Manufactured from stamped steel and approved for use on above grade floors. The box shall be 14" L x 8-9/16" W x 2-1/2" H [356mm x 217mm x 64mm]. Provide the box with two (2) independent wiring compartments that allow capacity for up to two (2) duplex receptacles, communication, and audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Provide each of the two (2) compartments with a minimum wiring capacity of 45 cu in [737ml3]. Two (2) compartments shall have a minimum of three (3) inches [76mm] of space behind the device plates. The box shall include the following number of conduit knockouts: four (4) 3/4-inch [19.1mm], eight (8) 1-inch [25mm] and two (2) 1-1/4-inch [32mm]. The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment. Provide box with two (2) blank device plates to seal off wire and cable routing passageways.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish or one of the powder coated finishes and delete the other finishes.

          - Furnish box with FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [bronze] powder coat] finish.

        - RFBA2R25OG Series 2-Gang On-Grade Shallow Depth Floor Boxes with Rectangular Activations: Manufactured from stamped steel and painted with a fusion-bonded epoxy approved for use metal reinforcement bar and related accessories before encapsulation in concrete and be approved for use in on-grade floors. The box shall be 14" L x 8-9/16" W x 2-1/2" H [356mm x 217mm x 64mm]. Provide the box with two (2) independent wiring compartments that allow capacity for up to two (2) duplex receptacles, communication, and audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Provide each of the two (2) compartments with a minimum wiring capacity of 45 cu in [737ml3]. Two (2) compartments shall have a minimum of (3) inches [76mm] of space behind the device plates. The box shall include the following number of conduit knockouts: four (4) 3/4-inch [19.1mm], eight (8) 1-inch [25mm] and two (2) 1-1/4-inch [32mm]. The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment. Provide box with two (2) blank device plates to seal off wire and cable routing passageways.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish or one of the powder coated finishes and delete the other finishes.

          - Furnish box with FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [bronze] powder coat] finish.

        - RFBA2R30 Series 2-Gang Above Grade Standard Depth Floor Boxes with Rectangular Activations: Manufactured from stamped steel and approved for use on above grade floors. The box shall be 14" L x 8-9/16" W x 3" H [356mm x 217mm x 76mm]. Provide the box with two (2) independent wiring compartments that allow capacity for up to two (2) duplex receptacles, communication and/or audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Provide each of the two (2) compartments with a minimum wiring capacity of 55 cu in [901ml3]. Two (2) compartments shall have a minimum of three (3) inches [76mm] of space behind the device plates. The box shall contain the following number of conduit knockouts: eight (8) 3/4-inch [19.1mm], two (2) 1-inch [25mm] and eight (8) 1-1/4-inch [32mm]. The box shall have four (4) removable knockout plates that can be replaced with a 2-inch trade size conduit hub (2HUB). The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment. Provide box with two (2) blank device plates to seal off wire and cable routing passageways.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish or one of the powder coated finishes and delete the other finishes.

          - Furnish box with FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [bronze] powder coat] finish.

        - RFBA2R30OG Series 2-Gang On-Grade Standard Depth Floor Boxes with Rectangular Activations: Manufactured from stamped steel and painted with a fusion-bonded epoxy designed for use on metal reinforcement bar and related accessories before encapsulation in concrete and be approved for use in on-grade floors. The box shall be 14" L x 8-9/16" W x 3" H [356mm x 217mm x 76mm]. Provide the box with two (2) independent wiring compartments that allow capacity for up to two (2) duplex receptacles, communication and/or audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Provide each of the two (2) compartments with a minimum wiring capacity of 55 cu in [901ml3]. Two (2) compartments shall have a minimum of three (3) inches [76mm] of space behind the device plates. The box shall include the following number of conduit knockouts: eight (8) 3/4-inch [19.1mm], two (2) 1-inch [25mm] and eight (8) 1-1/4-inch [32mm]. The box shall have four (4) removable knockout plates that can be replaced with a 2-inch trade size conduit hub (2HUB). The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment. Provide box with two (2) blank device plates to seal off wire and cable routing passageways.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish or one of the powder coated finishes and delete the other finishes.

          - Furnish box with FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [bronze] powder coat] finish.

        - RFBA4R25 Series 4-Gang Above Grade Shallow Depth Floor Boxes with Rectangular Activations: Manufactured from stamped steel and approved for use on above grade floors. The box shall be 14" L x 11-15/16" W x 2-1/2" H [356mm x 303mm x 64mm]. Provide the box with four (4) independent wiring compartments that allow capacity for up to four (4) duplex receptacles, communication, and audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Provide two sets of compartments of different sizes with the ones on the side of the box that are larger than the ones on the top and bottom. The large compartments have a minimum wiring capacity of 72.5 cu in [1,188ml3] and the small compartments have a minimum wiring capacity of 17.5 cu in [287ml3]. Four (4) compartments shall have a minimum of 2-3/4 inches [74mm] of space behind the device plates. The box shall include the following number of conduit knockouts: six (6) 3/4-inch [19.1mm], ten (10) 1-inch [25mm], two (2) 1-1/4-inch [32mm] and two (2) 1-1/2-inch [38mm]. The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish or one of the powder coated finishes and delete the other finishes.

          - Furnish box with FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [bronze] powder coat] finish.

        - RFBA4R25OG Series 4-Gang On-Grade Shallow Depth Floor Boxes with Rectangular Activations: Manufactured from stamped steel and painted with a fusion-bonded epoxy approved for use metal reinforcement bar and related accessories before encapsulation in concrete and be approved for use in on-grade floors. The box shall be 14" L x 11-15/16" W x 2-1/2" H [356mm x 303mm x 64mm]. Provide the box with four (4) independent wiring compartments that allow capacity for up to four (4) duplex receptacles, communication, and audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Provide two sets of compartments of different sizes with the ones on the side of the box that are larger than the ones on the top and bottom. The large compartments have a minimum wiring capacity of 72.5 cu in [1,188ml3] and the small compartments have a minimum wiring capacity of 17.5 cu in [287ml3]. Four (4) compartments shall have a minimum of 2-3/4 inches [74mm] of space behind the device plates. The box shall include the following number of conduit knockouts: six (6) 3/4-inch [19.1mm], ten (10) 1-inch [25mm], two (2) 1-1/4-inch [32mm] and two (2) 1-1/2-inch [38mm].. The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish or one of the powder coated finishes and delete the other finishes.

          - Furnish box with FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [bronze] powder coat] finish.

        - RFBA425 Series 4-Gang Above Grade Standard Depth Floor Boxes with Rectangular Activations: Manufactured from stamped steel and approved for use on above grade floors. The box shall be 14" L x 11-15/16" W x 3" H [356mm x 303mm x 76mm]. Provide the box with four (4) independent wiring compartments that allow capacity for up to four (4) duplex receptacles, communication and/or audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Provide two sets of compartments of different sizes with the ones on the side of the box that are larger than the ones on the top and bottom. The large compartments have a minimum wiring capacity of 90 cu in [1,475ml3] and the small compartments have a minimum wiring capacity of 22.5 cu in [369ml3]. Four (4) compartments shall have a minimum of 2-3/4 inches [74mm] of space behind the device plates. The box shall contain the following number of conduit knockouts: ten (10) 3/4-inch [19.1mm], two (2) 1-inch [25mm], ten (10) 1-1/4-inch [32mm] and two (2) 1-1/2-inch [38mm]. The box shall have four (4) removable knockout plates that can be replaced with a 2-inch trade size conduit hub (2HUB). The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish or one of the powder coated finishes and delete the other finishes.

          - Furnish box with FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [bronze] powder coat] finish.

        - RFBA4R30OG Series 4-Gang On-Grade Standard Depth Floor Boxes with Rectangular Activations: Manufactured from stamped steel and painted with a fusion-bonded epoxy designed for use on metal reinforcement bar and related accessories before encapsulation in concrete and be approved for use in on-grade floors. The box shall be 14" L x 11-15/16" W x 3" H [356mm x 303mm x 76mm]. Provide the box with four (4) independent wiring compartments that allow capacity for up to four (4) duplex receptacles, communication and/or audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Provide two sets of compartments of different sizes with the ones on the side of the box that are larger than the ones on the top and bottom. The large compartments have a minimum wiring capacity of 90 cu in [1,475ml3] and the small compartments have a minimum wiring capacity of 22.5 cu in [369ml3]. Four (4) compartments shall have a minimum of 2-3/4 inches [74mm] of space behind the device plates. The box shall include the following number of conduit knockouts: ten (10) 3/4-inch [19.1mm], two (2) 1-inch [25mm], ten (10) 1-1/4-inch [32mm] and two (2) 1-1/2-inch [38mm]. The box shall have four (4) removable knockout plates that can be replaced with a 2-inch trade size conduit hub (2HUB). The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish or one of the powder coated finishes and delete the other finishes.

          - Furnish box with FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [bronze] powder coat] finish.

        - RFBA6R30 Series 6-Gang Above Grade Standard Depth Floor Boxes with Round Activations: Manufactured from stamped steel and approved for use on above grade floors. The box shall be 19-3/8" L x 16-1/4" W x 3-3/4" H [492mm x 412mm x 95mm]. Provide the box with six (6) independent wiring compartments that accept up to six (6) duplex receptacles, communication and/or audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Provide two (2) sets of compartments of different sizes with the ones on the side of the box that are smaller than the ones on the top and bottom. The large compartments have a minimum wiring capacity of 62.5 cu in [1,024ml3] and the small compartments have a minimum wiring capacity of 22.5 cu in [369ml3]. Six (6) compartments shall have a minimum of 2-1/2 inches [64mm] of space behind the device plates. The box shall contain the following number of conduit knockouts: twelve (12) 3/4-inch [19.1mm], four (4) 1-inch [25mm], ten (10) 1-1/4-inch [32mm] and two (2) 1-1/2-inch [38mm]. The box shall have four (4) removable knockout plates that can be replaced with a 2-inch trade size conduit hub (2HUB). The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish or one of the powder coated finishes and delete the other finishes.

          - Furnish box with Large FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [brass] [bronze] powder coat] finish.

        - RFBA6R30OG Series 6-Gang On-Grade Standard Depth Floor Boxes with Rectangular Activations: Manufactured from stamped steel and painted with a fusion-bonded epoxy designed for use on metal reinforcement bar and related accessories before encapsulation in concrete and be approved for use in on-grade floors. The box shall be 19-3/8" L x 16-1/4" W x 3-3/4" H [492mm x 412mm x 95mm]. Provide the box with six (6) independent wiring compartments that accept up to six (6) duplex receptacles, communication and/or audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Provide two (2) sets of compartments of different sizes with the ones on the side of the box that are smaller than the ones on the top and bottom. The large compartments have a minimum wiring capacity of 62.5 cu in [1,024ml3] and the small compartments have a minimum wiring capacity of 22.5 cu in [369ml3]. Four (4) compartments shall have a minimum of 2-1/2 inches [64mm] of space behind the device plates. The box shall contain the following number of conduit knockouts: twelve (12) 3/4-inch [19.1mm], four (4) 1-inch [25mm], ten (10) 1-1/4-inch [32mm] and two (2) 1-1/2-inch [38mm]. The box shall have four (4) removable knockout plates that can be replaced with a 2-inch trade size conduit hub (2HUB). The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish or one of the powder coated finishes and delete the other finishes.

          - Furnish box with Large FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [brass] [bronze] powder coat] finish.

        - RFBA10R55 Series 10-Gang Above Grade Standard Depth Floor Boxes with Rectangular Activations: Manufactured from stamped steel and approved for use on above grade floors. The box shall be 19" L x 12-3/16" W x 6-1/4" H [483mm x 310mm x 159mm]. Provide the box with ten (10) independent wiring compartments that accept up to ten (10) duplex receptacles, communication and/or audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Provide the box with ten (10) independent wiring compartments of equal sizes that have a minimum wiring capacity of 22.5 cu in [369ml3]. Ten (10) compartments shall have a minimum of 2-1/2 inches [64mm] of space behind the device plates. The box shall contain the following number of conduit knockouts: four (4) 3/4-inch [19.1mm], seven (7) 1-inch [25mm], three (3) 1-1/4-inch [32mm], five (5) 1-1/2-inch [38mm], and three (3) 2-inch [51mm]. The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish or one of the powder coated finishes and delete the other finishes.

          - Furnish box with Large FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [brass] [bronze] powder coat] finish.

        - RFBA10R55OG Series 10-Gang On-Grade Standard Depth Floor Boxes with Rectangular: Manufactured from stamped steel and painted with a fusion-bonded epoxy designed for use on metal reinforcement bar and related accessories before encapsulation in concrete and be approved for use in on-grade floors. The box shall be 19" L x 12-3/16" W x 6-1/4" H [483mm x 310mm x 159mm]. Provide the box with ten (10) independent wiring compartments that accept up to ten (10) duplex receptacles, communication and/or audio/video services. The box shall permit tunneling from end power compartment to end power compartment. Ten (10) compartments shall have a minimum wiring capacity of 22.5 cu in [369ml3] and have a minimum of 2-1/2 inches [64mm] of space behind the device plates. The box shall contain the following number of conduit knockouts: four (4) 3/4-inch [19.1mm], seven (7) 1-inch [25mm], three (3) 1-1/4-inch [32mm], five (5) 1-1/2-inch [38mm], and three (3) 2-inch [51mm]. The box shall be fully adjustable, providing a maximum of 1-3/8-inch [35mm] pre-pour adjustment, and a maximum of 3/4-inch [19.1mm] after-pour adjustment.

          - The box shall accept RFBA device plates that are compatible with Legrand AVIP, Altinex Intera, Extron AAP, MAAP, Decorator Devices, and Ortonics® Series II audio/video devices.

Specifier Note: In the following subparagraph, select either the clear coated brushed aluminum finish, one of the plated finishes or one of the powder coated finishes and delete the other finishes.

          - Furnish box with Large FloorPort activation covers with [clear coated brushed aluminum] [[black] [nickel] [gray] [brass] [bronze] powder coat] finish.

      - ACTIVATION COVERS

        - FloorPort FPCT, FPBT, FPCTC, FPBTC and FPFFT Series Covers: Manufactured of die-cast aluminum or die-cast zinc, and available in brushed aluminum finish and powder-coated paint finishes (black, gray, bronze, nickel, and brass). Activation covers shall be available in flanged and flangeless versions. Covers shall be available with options for tile or carpet inserts, or flush covers. The cover’s hinge shall allow for the cover to open 180 degrees. The furniture feed covers shall come equipped with one (1) 1-inch trade size screw plug opening and one (1) combination 1-1/4-inch and 2-inch trade size screw plug.

          - Flanged covers shall be 7-3/4" L x 6-9/16" W [197mm x 167mm].

          - Flangeless covers shall be 6-3/4" L x 5-9/16" W [171mm x 142mm].

        - Large FloorPort FPLCT, FPLBT, FPLCTC, and FPLBTC Series Covers: Manufactured of die-cast aluminum or die-cast zinc, and available in brushed aluminum finish and powder-coated paint finishes (black, nickel, gray, brass, and bronze). Activation covers shall be available in flanged and flangeless versions. Covers shall be available with options for tile or carpet inserts, or flush covers. The cover’s hinge shall allow for the cover to open 180 degrees.

        - Large FloorPort FPLCTC[BK][BZ][BS][NK][GY][AL]TR Series Tamper-Resistant Cutout Cover Assembly: Manufactured of die-cast aluminum and available in powder-coated paint finishes (black, bronze, brass, nickel, and gray) and brushed aluminum finish. Flanged for use with six and 10-gang RFBA floor boxes; accepts tile or carpet inserts. 300 lb. load capacity. cULus Listed and RoHS Compliant.

        - Large FloorPort FPLBTC[BK][BZ][BS][NK][GY][AL]TR Series Tamper-Resistant Blank Cover Assembly: Manufactured of die-cast aluminum and available in powder-coated paint finishes (black, bronze, brass, nickel, and gray) and brushed aluminum finish. Flanged for use with six and 10-gang RFBA floor boxes; accepts tile or carpet inserts. 300 lb. load capacity. cULus Listed and RoHS Compliant.

      - FLOOR BOX ACCESSORIES

        - 2G and 4G Furniture Feed Barrier Kit: Catalog No. RFBA24FF; used to separate power and communication services in combination with Catalog No. FPFFTC furniture feed cover.

        - 2-Inch Conduit Transition Hub: Catalog No. 2HUB; 2-inch die cast zinc conduit fitting that allows for 2-inch conduit to transition into a shallow box design. Includes die cast fitting, gasket, and mounting screws.

        - Internal Duplex Device Plate: Catalog No. RFBADP; accepts standard 15- and 20-amp duplex receptacles.

        - Internal Tamper Resistant Duplex Device Plate: Catalog No. RFBADP20TR[BK][W]; pre-wired with tamper-resistant 20-amp PlugTail® receptacle; [black] [white] receptacle color. cULus Listed, RoHS and NAFTA compliant.

        - Internal Tamper Resistant Decorator Duplex Device Plate: Catalog No. RFBADEC20TR[BK][W]; pre-wired with tamper-resistant 20 amp decorator PlugTail® receptacle; [black] [white] receptacle color. cULus Listed, RoHS and NAFTA compliant.

        - Internal Blank Device Plate: Catalog No. RFBAB; closes off unused gangs.

        - Internal Decorator Style Device Plate: Catalog No. RFBADEC; accepts decorator style receptacles, communication, or audio/video devices.

        - Internal Single Device Plate: Catalog No. RFBAS1; contains one (1) 1-3/8-inch [35mm] receptacle opening.

        - Internal Single Device Plate: Catalog No. RFBAS2; contains one (1) 1-9/16-inch [40mm] receptacle opening.

        - Two-Gang Adapter Plate: Catalog No. RFBA10-2G; provides extra space for AV accessories; breakaway divider for larger accessories; cULus Listed, RoHS and NAFTA compliant; fits 10-gang box.

        - Three-Gang Adapter Plate: Catalog No. RFBA10-3G; provides extra space for AV accessories; breakaway divider for larger accessories; cULus Listed, RoHS and NAFTA compliant; fits 10-gang box.

        - Internal Communications Plate: Catalog No. RFBA6A; accepts bezels for common communications devices including TracJack, Series II, Keystone, and Lucent.

        - Internal MAAP Device Plate: Catalog No. RFBAMAAP; accepts two (2) Legrand AVIP or Extron Electronics MAAP Series Devices.

        - Internal AAP Plate: Catalog No. RFBAAP; accepts Extron Electronics AAP Series devices.

        - 4-Inch Rectangular Extension Ring: Catalog No. RFBA24R4EXT; extends height of floor box; compatible with 2- and 4-gang RFBA floor boxes.

        - 4-Inch Large Rectangular Extension Ring: Catalog No. RFBA10R4EXT; extends height of floor box; compatible with 6- and 10-gang RFBA floor boxes.

        - Rectangular 1/2" Collar: Catalog No. RFBA24R05EXT; extends 0.5” above surface of RFBA floor box; compatible with 6- and 10-gang RFBA floor boxes.

        - Bare Concrete & Terrazzo Ring: Catalog No. FP-CTR; nonmetallic ring for use on bare polished concrete and terrazzo floor applications; compatible with 2- and 4-gang RFBA floor boxes.

- EXECUTION

      - EXAMINATION

Specifier Note: Delete either of the options in the paragraph below to suit project requirements.

        - Examine conditions under which boxes and fittings are to be installed and substrate that will support boxes. Notify the [Architect/Engineer] [Construction Manager] in writing of conditions detrimental to proper completion of the work. Do not proceed with work until unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Strictly comply with manufacturer's installation instructions and recommendations and approved shop drawings. Coordinate installation with adjacent work to ensure proper clearances and to prevent electrical hazards.

        - Mechanical Security: Raceway systems shall be mechanically continuous and connected to all electrical outlets, boxes, device mounting brackets, and cabinets, in accordance with manufacturer’s installation sheets.

        - Accessories: Provide accessories as required for a complete installation, including insulated bushings and inserts where required by manufacturer.

        - Unused Openings: Close unused box openings using manufacturer's recommended accessories.

        - Provide a minimum concrete pour depth of 2-1/2-inch [64mm] flush to the top of the box for the RFBA2 and RFBA4 Series Shallow Boxes without collar and 3-inches [76mm] flush to the top of the box for the RFBA2 and RFBA4 Series Standard Boxes with collar. Provide the box with four (4) locations to accommodate leveling for pre-concrete pour adjustment and include four (4) leveling screws for the pre-pour adjustment.

      - CLEANING AND PROTECTION

        - Clean exposed surfaces using non-abrasive materials and methods recommended by manufacturer.

        - Protect boxes and fittings until acceptance.

END OF SECTION

© 2024 Legrand/Wiremold. All Rights Reserved.

A copyright license to reproduce this specification is hereby granted

to non-manufacturing engineers, architects, and specification writers.

Revd 02/24